[image: image3]
 SHAPE \* MERGEFORMAT 



[image: image4.png]7
“yitina



       常 州 金 马 包 装 机 械 有 限 公 司

         CHANGZHOU JINMA PACKAGING MACHINERY CO.,LTD 
[image: image5.jpg]/2"’
/7/171;17

& D AR







MT100型全自动圆瓶不干胶贴标机

1、概述

 MT100型圆瓶不干胶贴标机是由步进电机驱动的出标机构、框架机构、输送机构等组成。适用于圆形容器的不干胶标签的贴标，可一次完成贴标和收卷。本机采用微电脑可编程控制器进行控制，无复杂的参数设定，一键式操作，上手快捷，维护方便，是便捷型圆瓶自动贴标的首选设备。

[image: image1.jpg]



2、设备特点 

2.1触摸式人机界面和可编程控制器，功能丰富且操作简便直观。

2.2具有自动检测标签尺寸的功能，当检测到标签异常时，系统自动提示报警。

2.3可预置生产数量，累计生产量达到设定值时自动停机，方便生产控制和统计。

2.4输送、分瓶、贴标、打码、滚标功能均可单独控制和设定，应用灵活，对无须贴标的瓶子可仅作输送使用。

3、主要技术参数

3.1、适用瓶径：Φ35-200毫米（可根据客户要求定制）

3.2、瓶    高：80-300毫米

3.3、标签高度：20-180毫米（可根据客户要求定制）

3.4、贴标精度：±1毫米

3.5、生产能力：40瓶／分钟（依据标签长度不同）

3.6、电    源： ～220伏、50赫兹

3.7、总 功 率：0.5KW

3.8、重    量：约200公斤

3.9、外形尺寸：2000×1150×1350毫米（长×宽×高）

MT100 fully automatic round bottle self-adhesive labeling machine

1 Introduce：

  MT100 round bottle self-adhesive labeling machine is composed of a labeling mechanism, a frame mechanism, a conveying mechanism, etc. driven by a stepper motor. It is suitable for labeling self-adhesive labels on round containers, and can complete labeling and rolling at one time. This machine is controlled by a microcomputer programmable controller. It has no complicated parameter settings and one-button operation. It is quick to use and easy to maintain. It is the first choice for automatic labeling of convenient round bottles.

[image: image2.jpg]



2. Equipment characteristics

2.1 Touch human-machine interface and programmable controller, rich in functions and easy and intuitive to operate.

2.2 It has the function of automatically detecting the label size. When abnormal labels are detected, the system automatically prompts an alarm.

2.3 The production quantity can be preset, and the machine will automatically stop when the cumulative production volume reaches the set value, which facilitates production control and statistics.

2.4 The conveying, bottle dividing, labeling, coding, and rolling labeling functions can be controlled and set independently, and the application is flexible. Bottles that do not need labeling can only be used for conveying.

3. Main technical parameters

3.1. Applicable bottle diameter: Φ35-200 mm (can be customized according to customer requirements)

3.2. Bottle height: 80-300 mm

3.3. Label height: 20-180 mm (can be customized according to customer requirements)

3.4. Labeling accuracy: ±1 mm

3.5. Production capacity: 40 bottles/minute (depending on label length)

3.6. Power supply: ~220V, 50Hz

3.7. Total power: 0.5KW

3.8. Weight: about 200 kg

3.9. Overall dimensions: 2000×1150×1350 mm (length×width×height)

金马包装机械











常州金马包装机械有限公司


MT100圆瓶贴标机说明书














地址：江苏省常州市金坛区华丰路68号                  邮编：213200

电话：0519－68088386                                       传真：0519－68088396


