[image: image5]
 SHAPE \* MERGEFORMAT 



[image: image6.png]7
“yitina



       常 州 金 马 包 装 机 械 有 限 公 司

         CHANGZHOU JINMA PACKAGING MACHINERY CO.,LTD 
[image: image7.jpg]/2"’
/7/171;17

& D AR







MX304全自动抓盖式伺服旋盖机

①概述：

MX304全自动抓盖式旋盖机广泛应用于各类需要旋盖的产品，整机由抓盖旋盖机构、理盖输盖机构、电气控制系统和机架组成，采用直列式结构，伺服旋盖，实现了灵活、高效的旋盖作业。具有运行稳定可靠、包材适用范围广、品规更换快捷、操作维护简单等特点，适用于食品、日化、润滑油等行业各类型小口瓶盖的自动旋盖。

[image: image1.jpg]_,‘h
-

{
a7




②特点：

2.1采用最新式供盖、输瓶和伺服旋盖技术，省掉了复杂的机械结构，整机更简洁、美观且灵活易用；搭配高品质人机界面，绝大多数调整在触摸屏中参数设定即可。

2.2气动三爪式抓盖结构和伺服驱动速度力矩模式，先抓盖再旋盖，旋盖扭矩一致，保证旋盖适当的紧度且不损伤瓶和盖子，将最好的产品外观呈现到市场上。

2.3上盖理盖输盖机构整体式设计，可换式送盖模具，供盖系统更紧凑；旋盖机构高度电动升降调节、快拆式旋盖头、对应不同盖子和瓶型可实现快速切换调整；

[image: image2.jpg]



2.4产量计数功能，达到设定产量时自动停止。

2.5无瓶不旋盖，缺盖自动报警，输出堵瓶自动暂停，堵瓶解除自动恢复运行。

③主要技术参数：

3.1旋盖头数：1个

3.2旋盖速度：1200-1500瓶/小时
3.3适用包材：直径50－180mm，高度150－320mm
3.4盖子尺寸：直径25－50mm，高度15－35mm
3.5电源功率：220V/50HZ 6.7KW

3.6主机重量：600KG

3.7旋盖合格率：99.3%
3.8主机尺寸：1800（L）*1650（W）*2000（H）mm

MX304 fully automatic cap grabbing servo capping machine

①Introduction:

MX304 fully automatic capping and capping machine is widely used in various products that require capping. The whole machine consists of a capping and capping mechanism, a cap sorting and conveying mechanism, an electrical control system and a frame. It adopts an in-line structure and a servo rotary capping machine. cap, realizing flexible and efficient capping operation. It has the characteristics of stable and reliable operation, wide application range of packaging materials, fast product specification change, simple operation and maintenance, etc. It is suitable for automatic capping of various types of small-mouth bottle caps in food, daily chemical, lubricant and other industries.

[image: image3.jpg]_,‘h
-

{
a7




②Features:

2.1 It adopts the latest cap supply, bottle feeding and servo cap screwing technology, eliminating the need for complex mechanical structures. The whole machine is more concise, beautiful and flexible and easy to use. It is equipped with a high-quality human-machine interface, and most adjustments are made in the touch screen parameter settings. Just set it.

2.2 Pneumatic three-claw cap grabbing structure and servo drive speed and torque mode, first grab the cap and then screw the cap, the capping torque is consistent, ensuring the appropriate tightness of the cap without damaging the bottle and cap, and presenting the best product appearance to the market superior.

2.3 The upper cap sorting and cap conveying mechanism has an integrated design and a replaceable cap feeding mold, making the cap supply system more compact; the height of the capping mechanism can be adjusted electronically and the quick-release capping head can quickly switch and adjust corresponding to different caps and bottle types. 

[image: image4.jpg]



2.4 Output counting function, automatically stops when the set output is reached.

2.5 No capping when there is no bottle, automatic alarm for lack of cap, automatic pause in output of blocked bottles, and automatic resumption of operation when blocked bottles are released.

③Main technical parameters:

3.1 Number of capping heads: 1

3.2 Capping speed: 1200-1500 bottles/hour

3.3 Applicable packaging materials: diameter 50-180mm, height 150-320mm

3.4 Cover size: diameter 25-50mm, height 15-35mm

3.5 Power supply: 220V/50HZ 6.7KW

3.6 Host weight: 600KG

3.7 Capping pass rate: 99.3%

3.8 Host size: 1800 (L) * 1650 (W) * 2000 (H) mm

金马包装机械











常州金马包装机械有限公司


MX304全自动伺服旋盖机说明书














地址：江苏省常州市金坛区华丰路68号                  邮编：213200

电话：0519－68088386                                       传真：0519－68088396


